# Everyone Start Here

## Why do I need to read a manual for a keyboard?

The Model F keyboard, if properly set up and maintained, can provide you with decades of pure typing pleasure and performance, and it can hopefully be something you will use for the rest of your lifetime and maybe pass down to future generations.  Check out Chyrosran22’s excellent Model F overview video:  https://www.youtube.com/watch?v=y9Jds326gks

The trouble is, you need to read the manual and watch some step-by-step videos to learn the basics.  This kind of stuff is not something you can easily figure out on your own, even if you have used a Model F or Model M keyboard before.

Before you even touch your new Model F, please watch these short videos and review the manual, which is the quickest way to get everything up and running.  The time spent reading the manual will save you a lot of frustration and spending hours more than you need to.  Don’t be the one who starts installing keys and spends an hour trying to fix something that would take a minute if you read the manual.  Having to start over from the beginning because you forgot a step is a waste of time.  Please follow exclusively what is written in this manual – do not try to figure something out through hours of your own experimentation.  Most issues can be fixed within minutes, not hours.

If after you have read the manual you are unable to solve an issue, please post directly on the project thread on the Deskthority forum for support – please do not email me or post on the Q&A page for technical support.  The great communities of mechanical keyboard fans have been around for more than 10 years and have been very helpful to sort out all types of issues.  My primary focus is on continuing to mail out all the keyboards!  You are not alone and no one who buys a keyboard here will be left with a non-working keyboard.

The Model F keyboard is a robust design. Every part is 100% user-replaceable / user-repairable, often needing just a couple tools: screwdrivers, pliers, and at most a soldering iron. Compared to other consumer electronics products, Model F repair is easy and even a complete beginner can get up to speed quickly on how to use the keyboard software and keep their keyboard going for decades to come.

The goal of this community-oriented project is to give you a keyboard that you can learn to take care of yourself and that will serve you well for decades to come, long after production has ended, with help from the mechanical keyboard community on sites like Deskthority, geekhack, and reddit.  You will never be out of reach of someone who can offer you advice and help in the coming years.  Part of your responsibility is learning how to troubleshoot issues with the keyboard, which can come up at any time:  even right after your keyboard arrives after it has been bounced around in shipping! The most basic recommended maintenance involves just taking off the keys with a wire key cap puller to clean them with mild soap and water every now and then.  Please direct all questions and support requests to the project thread on the Deskthority forum.

Buying spare parts will help keep your keyboard operational in the decades to come.  Everyone please check out all the other items in the store. The most important extras are the First Aid Kit of spare parts for future repairs once production has long shut down (over 1000 kits ordered so far!), spare flippers with springs, spare barrels, key pullers, extra key sets (the key tops do wear down over time and you won’t be able to buy more XT quality one piece keys after the end of the project) and also a spare custom made 3 meter USB cable (same as the one that is installed with the keyboard). I hope to be able to manufacture as many First Aid Kits and spare parts as possible to keep all these great Model F keyboards running decades from now.

The current state of low-quality manufactured goods encourages a culture of just throwing something away or bringing it back to the store if anything is wrong with it.  For something complicated like a motherboard or graphics card that’s probably the best option, but the new Model F project philosophy is for the users to be able to fix small issues themselves due to the simplicity and full repairability of the Model F design (many buyers come from the world of the original IBM keyboards that are decades old and almost certainly require some maintenance work, so they expect it and are used to it). This keeps costs down so I am able to offer these keyboards at less than half of what IBM charged for them (adjusted for inflation).

The best way you can help this project:  If you like using Model F and Model M buckling spring keyboards, the best way to help the project is to tell other “tech-minded” people you know about the project. That would be greatly appreciated, and only if you don’t mind. And please do share some photos of your newly set up Model F Keyboard on the Deskthority and geekhack project threads if you’d like!

## Key (pun intended) features of the new Model F – why is it different from other keyboards

- The Model F technology features heavy duty metal parts contributing to the 5- to 10-pound weight of each keyboard, powdercoated cases for some original F’s (some 122, F50, F62, F77, F107, some XT and AT, 104 Unsaver). Metal top and metal bottom inner “sandwich” plates with interlocking tabs.  The Model F design is robust and easy to clean and self-repair with nothing more than a screwdriver, pliers, tweezers, and a key puller.

- High-quality parts contribute to the long life, with many Model F’s still in operation after decades of continuous usage. Still the best technology keyboard out there.  Will possibly outlast all other parts of your computer.

- Capacitive sensing buckling spring technology – long life enhanced by no metal on metal key contact

- Dye-sublimated PBT keycaps:  text does not wear off like most keyboards (pad printed, laser etched with filled paint, etc.)

- Individual injection molded barrels for each key made of premium plastic for enhanced smoothness and key travel

- Large injection molded capacitive flippers for a solid sounding click

- Birth certificate label on each keyboard – Label includes your keyboard’s exact birthday and serial number.

Model F Reproduction Keyboards specific features:

- Factory customizable with dozens of possible combinations of case color, key layout, key language, and custom birth date/serial number availability to choose from

- New available layouts not possible with the original Model F’s – HHKB style split right shift, split spacebar.  Can split keys like the original F’s (split back space, left shift, right shift, enter)

- Leverages the new open-source xwhatsit software and hardware built from the ground up as a Model F keyboard controller replacement

- QMK keyboard firmware now working on existing hardware of xwhatsit controllers

- Fully programmable xwhatsit Model F USB controller compatibility (ex-XT style keyboards) incl. powered by the Atmel ATmega32U2 chip

- Full NKRO (N Key Rollover)

- Parts made from brand new tooling and molds – like getting a Model F from the early 1980s right when they were introduced!

- Metal stabilizer tabs – Metal tabs attached to the top inner assembly for the Model M style spacebar stabilizer wire – upgraded from the plastic tabs in the original Model F keyboards

# Brand New Model F Keyboards Initial Setup and Troubleshooting Guide

## Safety precautions:

Always consult the booklet included with your new Model F keyboard for safety precautions and other important information.  Severe harm or even death can occur with any product if safety precautions are not followed.  Always use the right tools for the job – you will need a wire key puller to properly set up your new Model F keyboard as one or more keys may require more than one try to install it correctly.  Only use the USB cable that came with the keyboard, as not all USB cables are compatible or safe, especially the “smart,” Apple, or “E-Marked” USB-C cables with a PCB in the cable connector.

## Parts identification for Model F Keyboards

- Keys (from left to right:  Pebble, Pearl, Black, Industrial SSK Blue, 60% Dark Gray)

- Barrel

- Flipper with spring

- Top inner assembly

- Bottom inner assembly

- Inner foam

- Controller PCB with ribbon cable

- Large capacitive PCB (green or yellow)

- Top and bottom case

- Solenoid (optional)

- Solenoid driver (optional)

## Initial Setup

- Very important:  please read this entire section and go through everything step-by-step.  Otherwise, you will miss something and it could cost you hours of your time to fix it and you may even need to start over completely and take off all of the keys!

- First up, you need to inspect the keyboard’s flippers, springs, and layout.  Make sure you received everything you ordered and that nothing is damaged.  Part of a keycap may have snapped off in shipping.  Check all the various bags as parts may be mixed inside other bags.

  - Before installing any keys, make sure all springs on their flippers are present and move freely – a stuck flipper might require taking apart the keyboard if it cannot be loosened from the top side of the keyboard (if you have to open up and slide apart the keyboard’s inner assembly plates, be sure to remove the keys first!).  If you see a nub without a spring, the spring may have fallen off in shipping and needs to be replaced (refer to the key installation section for details, and then come back to this section).

  - You may notice some barrels without any flippers and springs.  This is intentional!  The extra wide keys have stabilizer inserts which go inside these barrels.  It is best to use new Model F stabilizer inserts with new Model F project keys and other stabilizer inserts with other keys.

  - It is also important to make sure the layout you ordered is the layout you received – refer to the product pages for details.  For key sets with full size Left and Right Shift keys, there should be no flipper and spring in the right barrel of the Left Shift, and nothing in the Left Barrel of the Right Shift.  For standard horizontal Enter (example:  US), there should be no flipper in the left barrel.  For ISO Enter, there should be no flipper in the top barrel of that key.

  - If your layout is not the one you wanted, stop now and go to the section on opening the inner assembly.  It is easy to change it – it only takes a few minutes.  The only thing you can’t change is going from HHKB-style split right shift to full size right shift, or vice versa.

- The next step is to install the stabilizer inserts:

  - Horizontal inserts (white stabilizers) are used for the 2U and wider keys in the barrels that do not contain a flipper and spring, with exceptions for two keys: the ISO Enter key uses the black/vertical stabilizer and the spacebar does not use stabilizer inserts.  You can’t put one in a barrel if there is a flipper or spring in that barrel already.

  - Push the insert all the way in the barrel so that it is flush with the top of the barrel. Install horizontal inserts (white inserts) as pictured below, with the “ears” on the left and right sides.  The vertical stabilizer inserts are installed also as pictured for the number pad 2U Vertical keys, but must be installed differently for ISO Enter!

  - The product photo shows each photo in the “right side up” orientation so for ISO Enter you’d just install the vertical (black) insert 180 degrees rotated from the photo, so that the larger “ear” of the insert is on the left.

  - If you mistakenly install the stabilizer incorrectly, or install the incorrect stabilizer, you will need to take apart the keyboard to push out the stabilizer insert from the other side.  An alternative to this would be to use straight tip lock ring / snap ring pliers, though not all of them would fit.  I personally use Wilde Tool G407 lock ring pliers to remove stuck inserts without having to take apart the keyboard.  Another alternative:  Check out this great method to safely remove horizontal and vertical stabilizer inserts without having to open up your new Model F keyboard to push out the insert!  https://deskthority.net/viewtopic.php?p=477651#p477651

  - Another way to remove a stuck stabilizer insert:  Quoted from a Model F owner: “If one of those little white plastic stabilizer inserts gets stuck in a barrel, a 1/4-20 inch thread tap (and tap wrench) will grip it well enough to get it out without damaging the barrel.”

- Next up is getting the spacebar right.  Model F keyboard spacebars are highly adjustable so you can dial in the kind of space bar you want, to some degree.  This is a key you will be pressing a lot so it is a good idea to get it right before proceeding to install the other keys.

  - The spacebar is the only factory preinstalled key with the keyboard but you may want to adjust it if it doesn’t sound or perform as you would like.  You can skip this section if everything is ok but it is tricker to adjust the space bar after you have proceeded to install all the keys.

  - Sometimes the spacebar requires adjustment after the keyboards are bounced around in shipping – sometimes they are bounced so much that people have found the spacebar separated from its keyboard barrel!  It’s a matter of slightly bending the spacebar so it performs optimally. Definitely more of an art than a science. When the spacebar gets stuck in my QC process, I usually take the wire with two hands, with the long part of the wire closest to my thumbs, and use both thumbs to very slightly push up the wire – I do this about 4-5 times but not too hard that the bend becomes noticeable. That way the wire is pushed out of the way enough of the metal stabilizer tabs so that there is a little room, maybe 0.5mm to 1mm. If they’re too close to the back of the metal tabs, the spacebar wire can get in the way of normal operation. This will also make the spacebar stabilizer wire ever so slightly wider in the two places where it connects to the spacebar itself – this also helps adjust the looseness vs. tightness of the spacebar while in operation. And as mentioned earlier, the metal spacebar tabs should not be pressed down so far that they cause the spacebar to get stuck, nor should they be parallel to the top inner assembly plate, where they may get stuck on the end of the spacebar.  Check if the spacebar wire is underneath the top of the metal tab – for both tabs!

  - If you need to separate the stabilizer wire from the spacebar, always hold the spacebar upside down and make sure the spacebar wire is flat against the spacebar; then you can pull the spacebar wire from one end in order to separate it from its hole in the spacebar.  Always remove the spacebar wire stabilizer from the spacebar before adjusting it.  If you just try to pull the wire straight off (or push on to) the spacebar, you may break the spacebar’s little tab.

  - The metallic twang / reverberation / ringing is definitely the sound I am going for (!) but there are ways of reducing it. The bigger the thud of the spacebar, the better in my view! In addition to the mods you referenced, one can also carefully push down the metal spacebar tabs for a reduction in rattling sound. Always push the side of the spacebar whose tab you want to adjust towards the metal tab in order to get that wire nearly touching the back of the metal tab before you push down that tab, then repeat for the other side. Pushed down even more and it may make the spacebar require “heavier” force to actuate (push down too much and the spacebar will get stuck frequently!), though this may result in some damage when trying to remove or adjust the spacebar.  “Applying a single layer of electrical tape where the stabilizer met the clips, as suggested by dotcom, helped immensely. I haven’t tried clickclack’s spacebar heat shrink mod but might do that someday. As it is, the spacebar is now the best sounding key on the keyboard.”

  - If the space bar is not actuating, test with a 1U key to make sure spring is good. Metal tab pressing down, scotch tape or heat shrink tubing application to reduce rattle if preferred. Please carefully remove and reseat the spacebar. This key’s tabs are easily broken so please be careful. For keyboards shipped in 2020 or earlier: if spacebar gets stuck, push down a little on the metal tabs which may have been pushed up a little too much when removing the spacebar.

  - If your spacebar lags a bit, just need to loosen the clips a touch with a screw driver, or you may need to bend the stabilizer wire a bit.  I believe that the nice spacebar thud sound (and minimizing rattle) is highly contingent on the proper placement of the metal tabs relative to the stabilizer wire– bending the stabilizer wire slightly away from the metal tab ends results in a more rattly spacebar with a lighter actuation force that some people prefer, while pushing too much towards the metal tab ends can slightly increase actuation force for the spacebar. Excellent spacebar stability and sound when the wire is directly against the back of the metal tab, or very close to it.  Another way of reducing the spacebar force slightly is to squeeze the stems of the key as described later on in the “wiggle method” section.

  - Regarding squeaky spacebars: that is nearly always the case of adjusting/slightly stretching or replacing the spring rather than due to the spacebar stabilizer wire. This is something I test on all keyboards as part of QC but I hope to improve on this. I recommend pressing down on the metal tabs if you re-seat the spacebar (after confirming no squeak and proper buckling when pressing the spacebar). A “squeaking” or stuck spacebar is often due to a bad spring combined with the need to slightly adjust the bend of the spacebar stabilizer wire – you can replace the spring without opening up the keyboard by using tweezers as shown in the QC secrets video above. Spacebar seating issues: Likely the spacebar tabs have been pushed down too much, or not enough. or the spacebar wire is bent out of shape (should be precisely rectangular). Also the spacebar should be installed as shown in the videos on the project blog (not entirely vertically or horizontally – I prefer a 45 degree angle with the spacebar row at the highest point of altitude).

  - Some prefer putting a clear plastic tube where the metal stabilizer wire touches the metal tabs.

  - One Deskthority member recommends automotive “silicone grease lightly applied (to the outsides of the barrels and the friction points for the stab wire).”

  - Another user:  “try putting thin adhesive neoprene under the stabilizer wire next to the tabs on the barrel frame. This has made the spacebar quieter and more solid, without making it mushy.”

  - To quote a DT member:  “Regarding spacebar rattle on the F62, I did several things, and the rattle has disappeared. Not sure which of these items worked or if it is the combination:  Put strips of self-adhesive rubber inside the spacebar. I used strips intended as non-skid feet for the undersides of various things, such as keyboards.  Applied a dab of silicone-base grease to the spacebar stabilizer clips on the top plate of the keyboard and on the spacebar itself.  Put strips of ultra-thin self-adhesive foam on the top plate of the keyboard where the spacebar stabilzer wire hits the top plate. I used Poron ThinStik polyurethane foam.  Put an O-ring at the base of the two spacebar barrels. The dimensions of the O-rings are 16mm OD, 12mm ID, 2mm thickness.”

- After that, you can install the keys.  Do not plug in the keyboard to your computer until instructed to!

  - The steps in this section will describe everything you need to know about how to install the keys, how to evaluate the sound of the spring as you test each key to make sure the key actuates properly when it is pressed, and how to reduce the buzzing sound of a key and/or fix a key whose spring does not click/buckle as it is pressed down.

  - It is completely normal for many keys not to function on your first try.  Removing and reinstalling a nonworking key a thousand times is not the way to go about setting up your new Model F!  Keep reading to avoid frustration and reduce your setup time astronomically.

  - Proper key installation involves holding the keyboard vertically, spacebar side up.

  - See my key installation video here to see how I install each key and make sure it is properly installed, but also see the next-linked video below on how to adjust things if the key does not actuate properly:  https://www.youtube.com/watch?v=xEm2mewsmrA

  - Don’t let the keyboard rest horizontally until each key has been fully pressed in.

  - Do not connect the keyboard to a computer until you have installed the keys.  Per a DT user: “When there are no keycaps, all the flippers will rest on the pcb giving you pressed keys all over.”

  - It is very important to test the Model F after you get it but before you start using it on your computer: Even though there is strong protective packaging, Model F springs are often dislodged during shipping which can result in a bad click sound or no click at all, and keys (and sometimes springs) may need to be reseated.  Test each key to make sure it buckles properly.  There is a Quality Control Secrets video embedded below that will show you step by step how to fix a nonworking key as well as help you determine what keypress sounds good vs. what is a potential issue.  The fix for each key takes less than one minute per key – you should not spend more time than that.  The last step is to replace the spring or switch it with another key’s spring – often springs may be damaged during installation and once damaged, a spring may no longer be usable.

  - The first keys you should install are the ones 2U and larger.  Due to the design of buckling spring keycaps, these keys are most likely to become stuck when pressed all the way down and require following these steps to fix such instances of key binding.

Best Method if the key is stuck:  you have to do some of the below steps several times in some instances of binding.  Do the steps, install the key and test with 10-20 key presses, then remove and do the steps again if needed.

Start with this step for keys with the stabilizer posts:  Gently pinch the affected key’s outer stem/ear and stabilizer insert pole between your thumb and forefinger and wiggle side to side about 20-30 times. Your fingers should be positioned so as to squeeze deeply into the key, as deep as possible, not just at the very end of the key stem.  A video on this fix:  https://www.youtube.com/watch?v=VBgQ9AO1Z_c

After squeezing, gently wiggle the key post (the part that goes into the stabilizer insert) 10 times in the direction of the left and right sides of the key if you were to look at the key as oriented on a keyboard – seems to eliminate binding when the extreme edges of a key are pressed.  Be very gentle as this is the easiest part to break on a key!  Do not touch the stabilizer insert itself or add any lubrication to it.

If it is still stuck but maybe a little better, repeat the prior two steps and test again.  Sometimes the two “wings” of the stem get stuck too close together so you have to gently spread them apart maybe a total of 1 mm or so.  For 2U height keys like 2U Vertical (num pad area) and ISO Enter, some folks have reported that the horizontal/white stabilizer insert works better with the vertical keys instead of the vertical/black insert.

Best Method if the key is too loose and pops off:  you just need to gently widen the stem a bit without breaking it. The stem consists of the two “legs” that go into the barrel.  It’s common for the larger key stems to be slightly too narrow or too wide. Please use the other keys’ stems for reference on the proper width.

Other potential fixes – not necessary or recommended:

Burnish back side of barrel with spring – use model m nut driver.

Keep stabilizer insert 1mm above top of barrel and there is no binding. Alternatively maybe add a 1mm piece of foam under side of key where stabilizer insert goes.  Example story:  “It seems like the left barrel (the empty one) on the spacebar was binding a bit. I solved it by cutting a thin foam disk (about 2mm thick) and sticking that into the outer barrel on the bottom of the spacebar. This limited the travel just enough to avoid binding.  Once that was done, the keyboard was perfect.”

  - And one last method that worked: “I did add a little bit of Syncro Superlube and that did it.”

  - Remove and re-seat any loose/non-working keys.  Regarding re-seating springs, in nearly all cases you do not need to take apart the keyboard to fix keys that do not click or spend a few minutes pressing a troublesome key.  Reattach the key as shown in these videos, with the keyboard positioned as shown in the videos (vertically, with the spacebar row up).

  - Often re-seating a key is not enough to make the key work or to reduce or eliminate a buzzing sound when a key is pressed, and you need to remove and reattach the spring as shown in the QC secrets video.  I have found that carefully removing and flipping the spring upside down can fix most spring issues, and replacing the spring with another spring is a last, but often necessary step.  Springs must always be removed with a gentle twisting motion (it should come off easily – otherwise you’re twisting the wrong way), never pulled straight up.  Twist counterclockwise looking at the spring straight down, like in the below picture.  The goal is (1) to have the spring touch the 12 o’clock position of the barrel when the keyboard is positioned that way, and (2) have the end of the spring in the 12 o’clock position relative to the flipper, as pictured below. If the spring end is not positioned at 12 o’clock (per the video) and the spring does not touch the barrel, buckling error is more likely to occur.

  - Here is the link to the Quality Control Secrets video showing how to remove and re-seat a key using key puller and fix a buzzing spring https://www.youtube.com/watch?v=Evn6vmrfD4M

  - Some folks report that after setting up the keyboard and connecting it to your computer to test each key with a keyboard tester program or web site, you should unplug the keyboard for at least 15 minutes while following the QC secrets video above to adjust any keys and springs that are flaky.  Sometimes unplugging the keyboard and plugging it back in fixes a random issue you may be having.

  - If the key doesn’t press because the spring can’t move, the flipper may be stuck.  Remove the key. Do the flipper and spring move freely in the barrel if you flick it?  Sometimes in shipping the keyboard is jolted so much that the flipper gets stuck and can’t move, and I have to carefully use tweezers to push the flipper back up into position to match the other flippers.  Sometimes moving the flipper carefully with tweezers does the trick. Otherwise, you have to open up the keyboard to reposition the flipper (there’s a 2-minute video on that process embedded later in the manual).  I am sure you checked every flipper to make sure it moved freely, before installing the first keycap, as noted earlier!  If you didn’t, now you may have to remove all the keys, open up the keyboard inner assembly, and reposition the flipper.

  - I strongly recommend tightening the two controller screws that ground the controller as they may have loosened as these keyboards are bounced around in shipping (if you see issues with holding down a modifier key and another key, or issues with key presses being out of sequence from what you typed when typing fast, fixing the grounding screws will likely solve the issue).  Do not use the Keypress Monitor part of the pandrew utility as it does not function correctly.

  - If the spacebar is making contact with the keys above it:  it is a matter of adjusting the shape of the spacebar stabilizer wire, and maybe putting some scrap foam or other material between the spacebar stabilizer wire and the back of the metal tab so that the stabilizer is moved upwards a bit – the spacebar needs to be moved farther away from the key it is making contact with, and that can be done by bending the spacebar and/or putting some material as noted above.  Do not sand away parts of the keycap or do anything else on your own as it is unnecessary.  “Putting a rectangular piece card stock vertically between the inner face of the tab and the wire did the trick” for one DT member.

  - There is definitely a break-in period with Model F keyboards – especially with the springs.  There’s a good chance that the springs will sound even better over time with usage!  Also pressing a key many times may fix an initially flaky key.

- Now that the keys are all installed and you tested each key several times before this step, we are going to connect the keyboard to the computer for the first time!

  - Do one more check before plugging in your keyboard.  Be sure you can hear the snap of the flipper when pressing each and every key.  I like to press each key a few times to be sure of a reliable actuation.  If you are just feeling the pressure of a spring when a key is pressed, but without a snap, that means you have not installed a key properly and many keys on the keyboard will not work, due to the way a pressed key messes up the autocalibration when a keyboard is plugged in.  Many errors are from folks with one improperly installed key and they think there is a major hardware problem, when in fact it is user error with installation.

  - Open a keyboard tester web site.  Be sure to plug the keyboard directly into the computer, not into a USB hub, and make sure there are no other connected USB devices besides the mouse (just for troubleshooting, to check for conflicts).  An important note on USB hubs and docking stations from one new Model F user: “Key presses would register or not register. Random text would appear while typing. Unusable.  Turns out I was using a 4 port non-powered Anker USB hub. Works fine for my model M’s, and the rest of my keyboards BUT not the F77.  I then plugged the F77 into a powered USB hub and tada, all is good.”

  - If the keyboard is not detected – check that the USB cable is plugged in fully on the controller end of the USB cable as well.  If you see “ATMEGA32 DFU” in your device manager (Windows), System Information (Mac), or “hardinfo” on Linux, you may have accidentally cleared the firmware and need to reflash it – see the advanced topics section below to fix this.   Also, try on another port and on another computer.

  - Test each key with the keyboard tester web site.  An important note:  refer to the product page for the factory default layout.  For example, the right Ctrl key may be factory programmed as the Fn key, so when you press this key it will not register as a Ctrl key and it will appear that the key is broken!

  - One less common issue you may find is inconsistent key registration – double presses, key ghosting, keys not recognized, key spamming (extra keys when one key is pressed).  Double presses and other issues could be related to the two controller ground screws not being tight enough, the seating of the spring on the flipper nub, or some kind of damage to the spring (the tops of the springs can get caught on something and bend out of shape – spring damage can’t be 100% fixed in my experience without replacing the spring itself), or the debounce filter and threshold (these two are xwhatsit firmware issues only-xwhatsit firmware stopped being used in early 2020).  This is usually an issue with the way that the keys were installed (did you follow every step so far? If the keyboard wasn’t held upright and each key tested for the proper click beforehand, that may be the cause). For example, you may see double presses if the spring is not properly reinstalled, especially if it is not pushed down all the way onto the flipper nub or if the spring is not reinstalled with its end between 12 and 1 o’clock.

  - When testing, always make sure the xwhatsit controller is attached to the gold-colored bottom inner assembly with both ground screws.  Otherwise, the keyboard will not function normally.

  - If, after you have followed the above instructions completely and many keys in a row or column do not work, please follow the section for additional diagnostics through the pandrew utility.  If a one or a few scattered keys do not work, it is most certainly not a hardware error but instead a user installation error or debris that entered the keyboard and moved between the flipper and capacitive PCB, thereby interfering with the keypress signal.

  - Does the rightmost row of keys get stuck when you press the key?  Sometimes all of the keys on the leftmost or rightmost side may be stuck because they are too close to the case.  Things may have shifted in shipping; all you need to do is loosen the 4 screws on the bottom and slide the mechanism away from the edge it is too close to, and of course tighten it back again when done.  Also check to make sure the inner plates slide in fully so the sides of the tabs touch the sides of the bottom inner assembly (see photo below).  If there is some space in between the tabs, then there could be issues with keyboard functionality.

## Next up, we will install the solenoid:

Safety warnings:  It is very easy to permanently damage the solenoid and driver by doing something another way.  If your solenoid does not work after installing it, you may need to update the firmware as a recent bug prevents the solenoid from working.  Most importantly, do not overtighten the screw that attaches the L bracket to your keyboard case or it might puncture the solenoid.  You may destroy your controller, solenoid driver, and/or solenoid by not following xwhatsit’s solenoid driver instructions as well as the below instructions.

The solenoid and required solenoid driver are add-on products not included with any keyboard.  There is a built-in firmware shortcut to easily turn on and off the solenoid so you don’t have to keep the solenoid on all the time! They can both be purchased through this link and are definitely recommended: https://www.modelfkeyboards.com/product/xwhatsit-beam-spring-or-model-f-controller/

- Follow this video, as well as the below instructions, to install your solenoid:  https://www.youtube.com/watch?v=Z0QNCkGcwhA

- Solenoid drivers help you add a solenoid to your xwhatsit-powered Model F or beam spring keyboard. You need both a solenoid driver and a solenoid; they are not sold together. The solenoid driver does not include the solenoid or screws but it does include the cable to connect to your controller.  When installing the solenoid, because the screw that secures the solenoid is at the bottom of the keyboard as shown below, you should use any of the slightly taller bumpers in the back (i.e., any option except the cork bumpers unless you double up and stack two of the cork bumpers, one on top of the other).  If you happen to use the 1″ large bumper and its large screw please do not drive the screw so much that it breaks in to the blue tape – it will destroy the solenoid.  You’ll need to do some experimenting to carefully do this.

- Why pick this solenoid over other solenoids or over IBM’s original solenoids? The new solenoids are far beefier than IBM’s originals, so your solenoid clicking will be even better! It’s a custom design that you can only order here. The specifications are 6v, 40 ohm, 0.15A, 0.9W, adjustable throw factory set to 1mm but with a user-adjustable range of about 0.5mm to 5mm (there are 2 screws that can be loosened to adjust the distance of the strike bar). The factory throw setting allows you to start using this solenoid with no modification (still need to follow xwhatsit’s solenoid installation guide to avoid damage to equipment). This solenoid has the dark blue outer covering, black and white twisted wires and 2.54mm connector like the original IBM solenoids, but with a strike bar so it won’t exactly resemble the IBM solenoid. This solenoid is not the one that’s available on eBay, though it looks similar and the voltage and resistance specs are the same. Also that other model doesn’t have the adjustable throw – you would have to spend time milling the strike bar to modify it for correct usage with the solenoid drivers. If the ebay solenoid’s throw distance isn’t changed, your solenoid will be very sluggish and won’t be able to operate at full typing speed. Also my solenoid is even larger than the eBay solenoid, the 26mm height dimension on eBay is set to be ~29-30mm on my solenoid (the solenoid goes sideways in F62/F77 so it fits) though all the solenoid specs are subject to change. With a larger metal frame, my solenoid should be a little louder than the eBay model and far louder than the original IBM! This is the largest possible solenoid that will fit inside the classic style F62 and F77 cases.   These solenoids are also a great option as an upgrade to IBM’s original solenoids too (I’d always save the originals of course for the sake of completeness).

- Most important is to match the square on both sides of the ribbon cable connector, so that you connect the squares (voltage line) to each other.  The position is different from what is pictured online, from other xwhatsit controllers, and from the photo below.  If you install the cable the wrong way you will destroy your equipment.  If unsure, do not connect it!  The square pad indicates VCC and that wire should go to the VCC square pin on the solenoid driver itself.  Connect square to square – voltage pin to voltage pin.

- Regarding the xwhatsit Model F expansion header pinout:  note that it is different from the original xwhatsit controller – you can damage the controller and other hardware by improper connections to the expansion pins.  Below is the pinout on the new Model F xwhatsit controllers.  Head over to the deskthority and geekhack forums for any and all questions related to the xwhatsit controllers.

- For the solenoid and solenoid drivers:  see the images below for the general installation procedure of solenoid drivers.  By default, the solenoids are not set to the maximum throw length:  to get the full solenoid experience, slightly loosen the two screws on the solenoid and move the bar as far out as it will go, then tighten the screws.

Adjust screws on solenoid, then slide out the bracket for maximum sound, so it looks like the below photo:

## Advanced solenoid configuration – only if using firmware files outside of the project web site.

Due to factory default QMK settings, the solenoid will only be under full power if the dwell time is set properly.  If you are using any of my hex files for either QMK or Via, this is done for you automatically.  However, if you use the QMK Configurator web site to make your firmware, import my JSON template file for your configuration and flash the firmware (see the firmware section of this manual), and while the keyboard is connected to your computer, hold down the keys Fn+Spacebar++= (the + = key next to backspace) which has been preprogrammed to increase the Solenoid dwell time by 1 ms.  Press it 20 to 25 more times to get it to the minimum required amount for full power.  You can also press the following options
Fn+Spacebar+T–>Toggle the Solenoid On/Off
Fn+Spacebar+ -_ Decrease Solenoid dwell time HPT_DWLD.  Refer to the two solenoid videos at the top of this page for a visual guide.  To program this into your own keyboard, for QMK drag the Any key to the key and layer you want, and then type in HPT_TOG to toggle, HPT_DWLI to increase the dwell time 1ms, or HPT_DWLD to decrease it.  Each key has to be in its own box of course.

## Using the pandrew utility to diagnose issues with many keys:

Download the following program file for Windows: QMK Model F Utility.   And here’s the Mac version of the pandrew utility.  Please note that the binary is not signed – you’ll need to “Open” manually in Finder.  Ctrl+double click maybe?  Per pandrew:  For “Linux – you have to build it:
git clone http://purdea.ro/qmk_firmware/
cd qmk_firmware/keyboards/xwhatsit/util/util
Read README.md for instructions on how to build”

With the keyboard in its normal horizontal position, plug in keyboard, load the utility program, and click Signal Level Monitor.  Then remove the keys at issue. Use your fingers to move the spring, to make sure it moves freely and make sure the flipper is not stuck.  The flippers would then freely make contact with the PCB. If you see the middle number of three numbers for each key being a solid green when the flipper is left alone, very close in numerical value to the bottom number for that key, that means there’s an issue with the installation of the key or of the spring – see if the flipper moves freely and the middle values change when the key is removed, if you rotate the keyboard vertical, spacebar side up, and then flat. Here’s how the matrix should look in the QMK utility after you have pressed each key one time and opened the Signal Level Monitor window in the pandrew utility program.  The keys that stay red represent pads without flippers (may vary based on layout).

If the middle row changes to green without an installed key, that means that you need to adjust the spring by following all of the steps and QC secrets video detailed earlier.  Removing and reseating the key again and again without doing anything else is unlikely to work.

If you see something like the below example (indicating a bad matrix row), then you’ll need to do some touching up of the solder as described above.  As you can see further below, the traces in the keyboard controller matrix do not correspond to actual rows of keys.

Now switch to the matrix view:  In the matrix view (screenshot below) you will see 3 pressed signals; these are not actual keys that are pressed.  There is one key in the 16th column:  the bottom right physical key on the keyboard. With this view you can more easily see if a row or column is damaged.

If many keys read “0” for the value, then there is possibly a short to ground and it is best to add a couple layers of insulating tape (polyimide/Kapton, electrical, Super 33, etc.) to the bottom of the controller where the ribbon cables are. This is more likely to happen to the ultra-compact cases as they are bounced around in shipping and the ribbon cable may break through the factory-installed tape.  If entire areas of the keyboard are not working and you are using an ultra compact case, sometimes the ribbon cable wire has broken through the Kapton / polyimide tape and is touching the case – one or more entire columns or rows would not function.  In this situation, you would see the problem go away when the keyboard inner assembly is outside of the case.  Also while the controller is installed and the keyboard is upside down with supports on either side (e.g. a book or something) so that the keys do not get pressed, push down on the part of the controller with the ribbon cable so that the ribbon cable end of the controller moves about 1-2 mm closer to the bottom inner assembly (gold color) plate – so as to move it away from making contact with the bottom of the case.

If the row or column reads a value higher than 0 (example: 110) but does not respond to key presses, then maybe the row or column needs more solder applied to the through hole, on the controller and/or on the capacitive PCB.  This also unfortunately does happen after these keyboards are bounced around in shipping, but the good thing is you don’t have to open up the inner assembly.  It is rare as I confirm each column and row before shipping, but it has been documented a few times. The controller is fine; you just need to add more solder to the holes for the ribbon connections for the missing column, on the PCB side of the components (see red rectangle in second photo below – doesn’t indicate the specific columns for you to solder – just a general photo). Just make sure you don’t touch any of the components themselves, just the column. No need to add solder on any of the other connections unless the connection looks like it needs more solder.

If the right ctrl on F62 or the right arrow key on the F77 (bottom right key) is not registering in the pandrew utility with the key removed, that usually means that column 16 needs to be retouched.  These keys are special in that only one key is in column 16.

Do two keys press when you press one key?  Occasionally there will be a solder bridge connecting two rows/column traces together, either somewhere on the PCB or specifically on the 30 through holes at the top of the board.  Just remove the bridged material carefully and then all will be well!

How do you find out which row or column needs more solder?  In the bottommost image below of the PCB traces, you can see the traces that the keys that don’t work are located.  For example, left alt, right alt, V, spacebar, M, the < , key, the key between right alt and right ctrl, and right ctrl are all on the same trace even though they are not on the same physical row.  You can follow the red line in the image and see that they are all connected to the leftmost trace, which is connected to the third through hole from the left, so that would be the area to touch up.  You need to add solder from the side of the controller components, not the bottom of the controller (just unscrew the two controller screws and gently bend the controller back as shown in the below photo).  Also you should carefully add solder to the large capacitive PCB (the other end of the ribbon cable for the same through hole you added solder to on the controller side).  Important note:  you should not open up the inner assembly for this procedure:  you should add solder from the side that you can see and access already (the underside of the large capacitive PCB shown in the below photo with the controller).  An important note that the row and column numbers shown in the controller image below should be disregarded for this purpose – just follow the traces and solder the one where all keys on that trace do not work.

For example, if the keys 2 Q W A S Z X and Left Alt do not work, then the second grouping of columns in the matrix below are at issue.  You can see that the 14th through hole from the left connects to the column that includes all those keys, so you would solder that through hole on both the front side of the controller (unscrew the controller as shown above) and if that doesn’t solve the issue, also add some solder to the accessible part of the large capacitive PCB-do not take apart the keyboard for this; just add from the bottom side that you can access with the inner assembly upside down.

# Intermediate Topics – Firmware, keyboard disassembly, changing cases

Here is the project’s YouTube channel which has a lot of videos on assembly and disassembly, quality control secrets, troubleshooting, solenoid installation, etc. https://www.youtube.com/channel/UCsi-1PcnCT3hw_RwcFzBnuw

## Opening up the keyboard inner assembly, changing case styles, etc.

- Make sure all the keys are removed from the keyboard before opening up the inner assembly!  See this two-minute video of mine:  https://www.youtube.com/watch?v=xaa23N9JDBs

- Keyboard disassembly note:  if you are not applying enough pressure on each screw, it is possible to strip the screws. With the correct driver and pressure, the screws should not be an issue.  If driver is not a tight fit, do not use the driver and purchase the correct driver-otherwise the screw will strip and you may not be able to remove the case except by drilling into the screw.

- Some layout changes are possible (open up inner assembly and move flippers around so they correspond with the layout you want) – e.g. change from ISO to ANSI or vice versa, split or unsplit the backspace, Left Shift, Right Shift, Enter, spacebar keys.

- You probably want to bend the larger tabs outwards a bit before clamping down as per one of the videos at the top of this page – this may make it easier to close!

- You can also change the original style case to another original style case, compact case to another compact case, or change from original to compact case and vice versa.  To change from the large case to the small case or vice versa: remove keys, open up keyboard case and bottom inner assembly while keeping keyboard upside down, then switch to the alternative bottom inner assembly metal plate.

- You can’t change HHKB style split right shift to standard or vice versa unless you purchase another PCB, top inner assembly, and inner foam as the keys are in slightly different physical location.

## Intermediate-level firmware adjustment

- While nearly all Model F keyboard users will stay with the default firmware and will never even consider reprogramming their keyboard to change the layout, the Model F keyboard does have a few powerful firmware options.  Only use the latest files and links I provide (all in one zip file for convenience; linked to in this section).  You can learn how to easily create a modified keyboard firmware file in minutes.  Do not use files downloaded previously from this page.

- Sometimes a key is not factory programmed as you’d want it to be, so you may just need to update the layout (see xwhatsit manual and/or QMK instructions on this page). If you so choose to update your firmware, please exclusively follow the QMK + Via customization video below to get started – do not follow any generic guides, use files provided elsewhere, or go to the generic home pages for these keyboard firmware files as they will not work.  Other QMK configurator pages will not work with Model F or beam spring keyboards.  The video description and firmware section below link to all of the files and resources you need.

- Safety note:  updating firmware is done at your own risk.  I have seen zero cases of damage from firmware updates but it may be possible.  From what I have seen, it is nearly impossible to brick the keyboard controller.  At the very worst you have to open up the keyboard to short PROG on the actual xwhatsit controller (unscrew the 2 controller screws to see the components side of the board, make a connection with a conductive metal object like an uncoated metal paper clip, then connect the USB cable to the controller and within a split second remove the paper clip).  Per pandrew:  “You might think that a screwdriver is gonna be enough, but due to surface oxidation it is likely not gonna be good enough. I like to use sharp metal tweezers, because they can dig into the pad a little bit, breaking up the surface oxidation, and I can actually be sure it’s shorting. Apply constant strong pressure on the pads while plugging in the keyboard to make sure they’re perfectly shorted. If you hear the windows USB plug-in sound (assuming the sound effect is enabled), then you know you have succeeded.  I’ve even seen people tell me they tried everything to short the pads with, and then I asked them to solder the pads together and it suddenly works. There’s something about the size of those pads, or about the fact that they need to be shorted in the perfect moment when the chip leaves reset, that make people think they are perfectly shorted and yet they aren’t. This affects me too, sometimes it takes me up to 3 tries to get it into bootloader mode.

- Another thing to double check that you are indeed shorting the PROG pad and not the reset pad. I believe in some versions of the wcass controller there is a reset pad on the opposite side of the board. So don’t be like ‘I see some pads, those must be it’, make sure to be shorting the correct ones. (I lost a couple of debugging hours in the past to someone shorting the wrong pads, that’s why I’m accentuating this point too.)”

- In under 16 minutes you can learn how to configure QMK or Via/Vial on your new Model F keyboard – just check out my step by step introductory video that shows exactly what to do:  https://www.youtube.com/watch?v=Esj2gZ0XptE

- If it is the first time loading firmware to your keyboard, follow the following steps which are not shown in the firmware video.  Then you can follow the video.  You may get an error “AtLibUsbDfu.dll not found” unless you follow these initial written steps which are not shown in the video.

- First enter the bootloader mode by pressing Left Shift + Right Shift + B if running QMK / Via, or if running xwhatsit’s old firmware, go into the utility, click Tools, Enter Bootloader.

- For your first time loading firmware, this is what you should see in your Windows Device Manager. If you don’t see it, keep trying the above steps to get into the bootloader mode.

- On Windows, install Atmel Flip 3.4.7 (see additional steps below for other operating systems) https://www.microchip.com/DevelopmentTools/ProductDetails/PartNO/FLIP In device manager, right click the “Atmel” device, update driver.

- Click browse my computer

- Click browse and browse to the installed location of your atmel software (may be in a different location than above). Make sure “include subfolders” is checked.

- This indicates successful install. Do not proceed unless all steps are successful.

- If you do not correctly go into bootloader mode in the above steps, you will see ibm-capsense-usb in your Devices and Printers menu. This is not good at this stage! You can’t proceed if you see this, so go back and try again.

- If you enter the bootloader mode correctly by following the previous steps, you will see ATmega32U2 DFU in your unspecified section of the Devices and Printers page. Only if you see this can you proceed.

- load atmel Flip 3.4.7, click the chip icon “select a target device”

- pick atmega32U2, click OK

- click the USB icon

- click USB

- click Open

- click File, Load HEX file

- Load the file eeprom eraser.hex. This file erases all of the configuration options as well as the firmware. Normal firmware updates do not erase the options. You want a full erasure.

- Click run, wait for it to finish. After this step, disconnect the USB cable from your controller, wait 10 seconds, then plug it back in. Then click the USB picture, click USB, click open like you did before.

- Now click file, load hex file again.

- Select the QMK or xwhatsit hex file you want to use.  DEPRECATED NOTE-for xwhatsit firmware, choose debounce 6 filter.hex. This file is available in the above xwhatsit firmware link.

- Make sure you see the name of the QMK or xwhatsit hex file in blue (shown correctly in the above screenshot). If not, load the firmware again. If there is no text below the “Hex file:” text, it means you are not flashing anything! Click run and wait for it to finish.

- Disconnect USB cable from controller.

- Wait 10 seconds, then plug back in.

- You should now see Brand New Model F Keyboards (QMK) or ibm-capsense USB (deprecated – xwhatsit firmware) in devices and printers window. If not and if you still see ATmega32U2 in this window, go back to the atmel flip and redo the steps from that point onwards.

## Currently there are 3 firmware options:

Link to all firmware files (QMK, VIA, xwhatsit original):  https://www.modelfkeyboards.com/wp-content/uploads/2020/07/QMK-layout-files.zip

Option 1:  QMK

QMK is the easiest option and is also the factory default option.

If you just want to switch the right side block or go back to the various “stock” firmware options, flash the desired firmware hex file from the above link and follow the below video – no need to follow most of the other written steps.

To customize the QMK firmware for your keyboard, follow this video of mine as well as the below steps: https://www.youtube.com/watch?v=Esj2gZ0XptE

- The beta QMK configurator is the only one that currently works with Model F keyboards – the main QMK Configurator site is not yet working with Model F keyboards.  Please join the Deskthority.net forum for any questions (they currently do not accept certain free email addresses such as Gmail).  When using the QMK configurator, always select the “wcass” controller version in the drop down list – do not select the universal or xwhatsit version as those are for other controller variations.  Beta QMK configurator link:  http://35.164.28.200:5000/#/xwhatsit/brand_new_model_f/f62/wcass/LAYOUT_all

- Here is the pandrew utility to send xwhatsit to bootloader with the click of a button

- And here’s the Mac version of the pandrew utility.  Please note that the binary is not signed – you’ll need to “Open” manually in Finder.  Ctrl+double click maybe?

- Per pandrew:  “Linux – you have to build it:
git clone http://purdea.ro/qmk_firmware/
cd qmk_firmware/keyboards/xwhatsit/util/util
Read README.md for instructions on how to build”

- Atmel Flip 3.4.7:  my preferred way to flash firmware for Windows computers https://www.microchip.com/Developmenttools/ProductDetails/FLIP

- Use the QMK Toolbox program to flash firmware on Apple/Mac computers.  https://github.com/qmk/qmk_toolbox/releases

- To see whether you are currently running QMK or xwhatsit firmware, go to Devices and Printers (Windows) and it should say ibm_capsense_util for xwhatsit firmware and “Brand New Model F Keyboard” for QMK firmware.  Keyboards I shipped starting around Summer 2020 onwards use QMK.

- pandrew’s Model F QMK documentation / manual can be found here:  https://deskthority.net/viewtopic.php?p=480196#p480196

- The full documentation for the QMK project (not just Model F keyboards) can be found here:  https://docs.qmk.fm/#/

- After you are on the beta QMK site:

  - Select the keyboard you have – either xwhatsit brand new Model F F62 wcass or F77 wcass.

  - Load one of my preprogrammed layout files, based on your keyboard configuration: same zip file as the link immediately above.

  - To change a key, go to the bottom of the page and select ANSI or ISO/JIS where needed.  Click with your left mouse and drag the needed key you want to change to the middle area of the page titled “KEYMAP:” in the position you want the new key to go – pay attention to being on layer 0 (base layer), layer 1 (function layer 1), etc.  I believe the “MO 1” key is the function key for function layer 1.

  - Then you click Compile, then wait until it’s done.  Then click download firmware.

  - pandrew QMK Model F utility to send xwhatsit to bootloader – also a good diagnostic utility (follow the firmware video at the top of the manual).  Very important to also follow the “Detailed firmware flashing instructions” section below – do not attempt to flash without following every step 100%.

  - QMK interesting feature – QMK with the Model F so “I can program the modifiers to be arrow keys on tap”

Option 2:  Via/Vial

A GUI program that you can run offline to adjust your keyboard.  Via is the closed source version and Vial is the open-source version.  It runs QMK firmware that is slightly modified to allow it to work in Via/Vial.  Please direct all questions related to Via to the project thread on the Deskthority forum.  Via Model F documentation is provided by darkcruix at www.bucklingspring.com and the full Via documentation can be found at https://caniusevia.com/

Via firmware is not loaded by default, so if you want it, follow the YouTube video above.  If you miss a step or do any steps out of order, it may not work!

Important note:  Select a firmware labeled “0-9” if you plan on assigning keys to the blank keys in the PrintScr layout.  Unfortunately, there is a bug where if you pick any layout labeled PrintScr, you cannot assign these two keys later on because they were not initially assigned.  This means you’ll have to change the right side block layout entirely yourself.  Details here

Here is an up to date step-by-step guide on flashing Vial, as of June 2022:  https://deskthority.net/viewtopic.php?f=7&t=27260

Current preprogrammed keypress commands in Via and QMK:

Hold down the keys Fn+Spacebar+
T–>Toggle the Solenoid On/Off Any key HPT_TOG
+= Increase Solenoid dwell time HPT_DWLI
-_ Decrease Solenoid dwell time HPT_DWLD
E–>EEPROM Reset (erases all settings like if you adjusted a dwell time for solenoid)
R–>Reset (enter bootloader)
D–>Debug

Command keys: hold Left Shift + Right Shift +
B–>Enter Bootloader
N–>Toggle N-Key Rollover (NKRO) (off by default to maximize compatibility)
Others: https://beta.docs.qmk.fm/using-qmk/advanced-keycodes/feature_command

Option 3:  xwhatsit firmware

xwhatsit firmware is now deprecated in favor of the newer QMK firmware which was ported over to the Model F xwhatsit controller in 2020.  To see whether you are currently running QMK or xwhatsit firmware, go to Devices and Printers (Windows) and it should say ibm_capsense_util for xwhatsit firmware and “Brand New Model F Keyboard” for QMK firmware.  Instructions and resources for this old firmware can be found in the appendix.

# Appendix (additional resources)

## Maintenance, cleaning, spills, noise dampening, TrackPoint Mods

- As noted in the Model F Project Philosophy, the Model F is designed to be your keyboard for decades, just like many of the originals still clicking after nearly 40 years!

- Model F Cleaning:

  - There are many Model F cleaning videos.  Always keep your keyboard unplugged during cleaning and while things are drying.  Always let your keycaps air dry overnight in a non-humid place with a good amount of air flow.  Never reinstall keycaps after only a few hours of drying as there will still be some small droplets that will interfere with keyboard operation later on.  Here’s the first result from a YouTube search:  https://www.youtube.com/watch?v=wPOe0Pmd3gs

  - The important thing is to use only mild dish soap and lukewarm water.  I clean my Model F and Model M keys and barrels, but not springs/flippers, in an ultrasonic cleaner just like what was done in the above video posted by someone else.  I almost never clean the flippers/springs unless there is a residue that cannot be removed with a dry cloth.  Always wait a day before putting the keys back on.  There are so many crevices inside the keys for water to get stuck and this will cause operational issues if they do not air dry.  A fan blowing on the keys, all on a large towel works well.  I gently clean the case with soap and water, and sometimes very carefully with a melamine sponge (be careful as this can take the paint right off and cause damage).

  - Don’t get the controller wet or you will damage it or potentially cause a fire.

- Liquid spill: First thing to do is take apart the keyboard.  You need to take it apart and dry out everything.  Spilling anything probably requires scrubbing each flipper and the PCB with mild dish soap and water and letting them air dry for a day.  This video will show you how to take care of this: https://www.youtube.com/watch?v=xaa23N9JDBs

- Floss mod:

  - https://geekhack.org/index.php?topic=9761.0

  - https://deskthority.net/wiki/Dental_floss_mod

  - https://deskthority.net/viewtopic.php?p=468281#p468281

  - https://geekhack.org/index.php?topic=79141.msg2941878#msg2941878

  - Key sound reduction discussion – floss mod, etc.: https://deskthority.net/viewtopic.php?p=463382#p463382

  - “Floss mod can increase key weight (especially if the floss is longish, but not long enough to prevent actuation).” https://deskthority.net/viewtopic.php?f=2&t=22175

  - Example floss mod YouTube sound test

  - Example floss mod YouTube 2022 test

- Grease mod/Soft Touch mod

  - In addition to the floss mod described on the Geekhack/Deskthority keyboard forums, another mod to consider is the grease mod, researched and described by the below YouTube user.  Also in my research I have found that pressing the springs down all the way on the flipper (normally there is a 0.2 to 0.3mm gap) reduces the ringing.  I do not offer and have not tried any of these mods but below is some information to consider.

  - The purpose of the grease mod is to reduce the reverberation/ringing after each key is pressed.

  - His comments are copied directly below.

  - Apply Synco Superlube on the inside of the spring with a metal probe…just enough to dampen the vibration

  - As an experiment, I tried a few switches on an XT F. I did nothing to the first XT F in the video…you can hear the sound I’m referring to…it’s like a super-loud, much more intense, sharper and “drier” version of what my (in superb condition) 1988-1989 Model M boards sound like. Then on the second board, I did the Alps spring lube trick on the keypad only. The feel is the same, the click sounds great (to me), the actuation force is ever so slightly less. But the annoying (to me) spring noise virtually disappears.

  - video 1 – detailed key sound (lubed keypad; unmodified other keys):  https://www.youtube.com/watch?v=Kha_cvhZBp8

  - video 2 – typing demo of the unmodified F:  https://www.youtube.com/watch?v=d1j1g-xLF6k

  - video 3 – typing demo of the fully spring lubed F:  https://www.youtube.com/watch?v=yqvr-U9CEuY

Dab of glue in barrel mod: (placed approximately in the spot of the number 7 mold cavity number below; so that the return of the flipper does not make as much noise)

Great series of videos showing different new Model F keyboard mods by DT user Twst (floss mod vs. no floss, buzzers, beepers, different solenoids, etc.):  https://www.youtube.com/channel/UCh7FbPSydoafCYMgLrCfk1w/videos

Trackpoint mod – adding a trackpoint to your New Model F keyboard

SUCCESS Trackpoint MOD!

Here’s a link to the photos from silentbob’s Brand New Model F trackpoint mod.

https://imgur.com/a/Sp7sfYO

“I used the Unicomp M13 trackpoint but the others I mentioned will also work. I used the unicomp controller and have two USB cables running out the back. If the current controller has any extra inputs that handle PS2 or some unused GPIO pins then it could be integrated into the existing controller with the stand alone SK8702 trackpoint controller. I plan on going bluetooth in which case I can join the two controllers through a hub internally and not have to worry about two controllers and cables. I used the Unicomp GHB keys with the cutout to clear the trackpoint.”

Additional non-Model F trackpoint guide:  https://deskthority.net/viewtopic.php?f=7&t=16643&start=

## Project Philosophy: Full user control of product maintenance and repairs

Goal:  the best quality product at the lowest price point, with a focus on the end user being able to stockpile spare parts as well as set up and fix their Brand New Model F.  There are definitely markets for hardware with service contracts. Not sure about a market for a +$100 more costly keyboard with full maintenance and technical support though. IBM’s 1980s price guides mentioned they would require charging banking customers a minimum of about $100 per year (not adjusted for inflation) for each original Model F keyboard in maintenance costs as part of a service contract. However, if I had to hire staff to deal with “free” returns, more personalized technical support / phone support, and doing even the most minor repairs (re-seating keys, replacing springs and barrels, changing the USB cable, etc.), each keyboard would cost a lot more because of overhead costs, and these keyboards are already not inexpensive to begin with. And it would slow me down even further mailing out these great keyboards. I believe that this direct to consumer, community type project is the best way to bring the Model F to as many people as possible and at the lowest possible cost.

Read more about the mission statement of the project on the About page:  https://www.modelfkeyboards.com/about/

Here is a Model F Keyboard NEW Kishsaver vs. original IBM keyboard sound comparison https://www.youtube.com/watch?v=uRlw7loUw7I

## Original Model F Topics; 12 Misconceptions about the Model F Keyboard – what initially scares many people away

- All the ways to make your IBM Model F work with USB

  - I strongly recommend using the original F122 controller with Soarer’s converter (plugs into the IBM controller). It’s what I’m using on all my F122 keyboards.  See the attached ZIP file here for some helpful tips gathered from my own experiences and from others.

  - Removing the original controller is not a good idea as the traces are at risk of damage during the desoldering process (has happened to me) and it is completely unnecessary for the F122. Controller removal is only required for the metal case Model F Keyboards (F50, F62, F77, F107).

  - Instead of a $25 teensy I also use an ATmega328 / ATmega32U4 style Pro Micro which is readily available under $10 shipped on eBay.

- What is the foam for, and do I need to replace it

  - The foam probably served two purposes: first to reduce the possibility of dust and debris entering the inner assembly and affecting the capacitive key sensing, and second, to reduce wobbling of barrels and ensure they are tightly against the capacitive PCB (the large PCB underneath all the keys).

  - New inner foam is strongly recommended to restore all Model F Keyboards (I have foam available for all Model F keyboards except the XT style). I recommend ordering an extra 1-2 pieces as the foam has a limited installed life (while compressed inside the keyboard it loses its resiliency over time and develops compression set, meaning the foam does not rebound to its original thickness). The extra foam should last for years if stored not in a keyboard, in a sealed plastic bag in a climate controlled area. Once production shuts down you won’t be able to purchase it and I’m the only maker of new factory cut Model F foam.

- 12 Misconceptions about the Model F Keyboard – what initially scares many people away:  https://www.modelfkeyboards.com/misconceptions-about-the-model-f-keyboard-what-initially-scares-many-people-away/

## A beginner’s guide to full configuration of QMK/Via:

Note:  this is completely unnecessary to do unless you want to change options that cannot be changed in the QMK GUI / Via GUI.  It is a local way of compiling the firmware through the command line to update the factory default solenoid dwell times, add in darkcruix’s patch for excluding certain keys for the solenoid, allow NKRO but disable it by default, etc.

The instructions from pandrew were very helpful but for a beginner such as myself, I needed step by step instructions which I have updated below. This will allow those with little to no programming knowledge to adjust the settings in QMK that cannot be adjusted from the QMK configurator GUI web site.

At the top of this section I provided links to download the latest JSON files and the latest firmware, which includes the above-mentioned changes.

Once again much thanks to pandrew and darkcruix for their outstanding work on QMK and Via.

To customize Via firmware beyond layout customizations (for example, to make all the above changes), please contact darkcruix, who put together a document listing changes in the code to get the firmware working with Via. You currently lose the ability to diagnose the controller in pandrew’s QMK utility if you use Via firmware. To just have Via with the default settings that cannot be changed in the Via program, please refer to the below notes.

New procedure from scratch Ubuntu install:
download and install Ubuntu in a Virtual Machine

open Terminal program and enter one line at a time:

sudo apt update && sudo apt upgrade -y
sudo apt install python3-pip
sudo python3 -m pip install qmk
echo “PATH=$PATH:$HOME/.local/bin” >> ./.bashrc
sudo apt install git
qmk setup
mkdir qmk_firmware_andrei
cd qmk_firmware_andrei
git clone http://purdea.ro/qmk_firmware/

do this action-not a command in terminal
copy keyboards/xwhatsit folder to qmk_firmware/keyboards folder
the xwhatsit folder has all the changes in QMK

code changes to make
config.h in the folder of the keyboard you want to update – for example, keyboards/xwhatsit/brand_new_model_f/f77/wcass
lines ~286-287
#define SOLENOID_DEFAULT_DWELL 20
#define SOLENOID_MIN_DWELL 20

comment out (put // in front of) line ~28:
//#define DESCRIPTION QMK firmware for the modelfkeyboards.com reproduction of the IBM Model F keyboards
comment out lines ~199-202:
//#ifndef LINK_TIME_OPTIMIZATION_ENABLE
// #define NO_ACTION_MACRO
// #define NO_ACTION_FUNCTION
//#endif

add to end of haptic.c file – new line:

#define HAPTIC_EXCLUSION_KEYS 1

in rules.mk:
NKRO_ENABLE = yes # USB Nkey Rollover
add this line at bottom:
LTO_ENABLE = yes # Link Time Optimization – reduces file size
HAPTIC_ENABLE = yes
HAPTIC_DRIVER = SOLENOID
(delete the original haptic_enable line in the rules.mk file before adding the new lines above)

Per BuccoBruce2, “copy the contents of the tmk_core directory from pandrew’s repo into your qmk_firmware folder. Do not overwrite any existing files – they’re probably newer.”

put the json file with your custom layout (the file you created in QMK configurator) in the root of the Home folder
command to set the keyboard – can change F77 to F62 in this example if you have an F62:

qmk config user.keyboard=xwhatsit/brand_new_model_f/f77/wcass
qmk compile “../1.json”
it will give a random name and place the file in the root qmk_firmware folder – not the name of the json file, so if compiling multiple layouts, rename the newly created hex file before proceeding to compile the next layout
../ means go up one folder from qmk_firmware folder

examples:
qmk config user.keyboard=xwhatsit/brand_new_model_f/f77/wcass
qmk compile “../F77_-_ANSI-ISO_-_0-9.json”
qmk compile “../F77_-_ANSI-ISO_-_PrintScr.json”
qmk compile “../F77_-_HHKB_2U_backspace_-_0-9.json”
qmk compile “../F77_-_HHKB_2U_backspace_-_PrintScr.json”
qmk compile “../F77_-_HHKB_split_backspace_-_0-9.json”
qmk compile “../F77_-_HHKB_split_backspace_-_PrintScr.json”
qmk compile “../F77_JIS_-_PrintScr_-_need_to_do_0-9_layout_and_add_function_layer.json”
qmk compile “../zF77_-_HHKB_split_backspace,_everything_else_ANSI_-_0-9.json”
qmk compile “../zF77_-_HHKB_split_backspace,_everything_else_ANSI_-_PrintScr.json”
qmk compile “../zF77_-_HHKB_Split_Shift,_everything_else_ANSI_-_0-9.json”
qmk compile “../zF77_-_HHKB_Split_Shift,_everything_else_ANSI_-_PrintScr.json”

to then switch to compiling f62’s:
qmk config user.keyboard=xwhatsit/brand_new_model_f/f62/wcass
qmk compile “../F62_-_ANSI-ISO_-_Regular_2U_or_Split_Backspace.json”
qmk compile “../F62_-_HHKB_-_Regular_2U_Backspace.json”
qmk compile “../F62_-_HHKB_-_Split_Backspace.json”
qmk compile “../zF62_HHKB_split_right_shift_only,_everything_else_ANSI.json”

to then switch to compiling f62 scumyc/scumnc:
qmk config user.keyboard=xwhatsit/brand_new_model_f/f62_scumyc/wcass
qmk compile “../zF62_scumnc_scumyc.json”

Additional changes for Via (quoted from darkcruix’s post and updated):

Initially the util_comm.c needs to be adjusted:
…
void raw_hid_receive_kb(uint8_t *data, uint8_t length) {
if (0 != memcmp(data, magic, sizeof(magic))) {
return;
}
…

One other change you may have to make to get the xwhatsit driver to build with the most recent QMK master snapshot is to edit util_comm.c and change…
CODE: SELECT ALL

#include <tmk_core/common/eeprom.h>
…to…
CODE: SELECT ALL

#include <platforms/eeprom.h>…due to that header file being recently relocated.

Within the specialized firmware itself the following adjustments are required:
config.h: – here: standard F62 variant
#pragma once
#include “config_common.h”
#undef PRODUCT
#undef MANUFACTURER
#undef DESCRIPTION
#undef PRODUCT_ID
#define MANUFACTURER Brand New Model F Keyboards
#define PRODUCT Brand New Model F Keyboards – F62
#define DESCRIPTION Brand New Model F Keyboards F62
#define VENDOR_ID 0x1209
#define PRODUCT_ID 0x4704
#define TAPPING_TERM 200
#define TAPPING_TOGGLE 2
#undef BOOTMAGIC_ENABLE
#undef BOOTMAGIC_LITE
#define BOOTMAGIC_LITE_ROW 0
#define BOOTMAGIC_LITE_COLUMN 0

(important – need to remove above items under USB Device Descriptor section – cannot have duplicates)

rules.mk:
VIA_ENABLE = yes
COMMAND_ENABLE = no
NKRO_ENABLE = no
LTO_ENABLE = yes
MOUSEKEY_ENABLE = yes
EXTRAKEY_ENABLE = yes # Audio control and System control
CONSOLE_ENABLE = no # Console for debug

Do not put TAP_DANCE_ENABLE = yes

Compiling/running the xwhatsit capsense utility on Linux:

For Ubuntu:
1. Open the Software & Updates program and select Community-maintained free and open-source software (universe)
2. Extract the precompiled file into a directory
3. Open a terminal and cd into that directory
4. In a terminal run: sudo apt update && sudo apt install gcc gcc-avr avrdude
5. sudo apt install build-essential
6. sudo apt install qtcreator
7. Run sudo ./ibm_capsense_usb_util

The below information was provided by Geekhack member joneslee85 to help with troubleshooting on linux:

* Problem: Linux config tool won’t build with GCC 6.x due to an
intentional breaking change in GCC’s handling of “-Isystem”.
One possible workaround: build the tool and all C++ dependencies with GCC 5.4.x

joneslee85 workaround:  I managed to workaround this issue by sub the -Isystem with -I:

Code:
sed -i ‘s/\-isystem \/usr\/include/\-I\/usr\/include/g’ Makefile

Below is the full code I do on Fedora 32:

Code:
sudo dnf install avr-gcc avr-libc qt5-devel hidapi-devel glibc-headers make gcc
wget https://static.wongcornall.com/ibm-capsense-usb/0.9.0/ibm-capsense-usb_0.9.0.tar.gz
tar xzvf ibm-capsense-usb_0.9.0.tar.gz
cd ibm-capsense-usb_0.9.0/src/util
qmake-qt5 ibm-capsense-usb-util.pro
#replace `-isystem /usr/include` with `-I/usr/include` # Ref: https://bugzilla.redhat.com/show_bug.cgi?id=1835441
sed -i ‘s/\-isystem \/usr\/include/\-I\/usr\/include/g’ Makefile
make

# binary is now in src/ibm_capsense_usb_util

How to compile the xwhatsit firmware hex file on Linux (easier to do than on Windows):

For Ubuntu:
Open the Software & Updates program and select Community-maintained free and open-source software (universe)
In a terminal, run: sudo apt update && sudo apt install make gcc gcc-avr avrdude avr-libc build-essential
To get patch files from GitHub, click on the commit and in the URL, append .patch and then save that as a .patch file. pandrew’s work to create 0.9.2 firmware can be found here: https://github.com/purdeaandrei/ibm_capsense_usb_mods/commits/master
Move the patch files into the src directory
In the terminal cd into the src directory
To change the files based on a patch file, run patch < filename.patch
Run patch files in order from oldest to newest (start with debounce patch, then patch 1, then patch 2, then patch 3 (the latter three patches from pandrew).
Run make and copy the ibm_capsense_usb.hex file that was created
The default debounce is 11. To create the debounce 6 version, edit the scan.h file and change the line with #define SCAN_DB_THRESH_TOP 11 – change to a 6 (I use the debounce 6 version when sending out the Brand New Model F keyboards)
Run make and copy the ibm_capsense_usb.hex file that was created

The open-source controller PCB files used in this project can be downloaded here:  updated_PCB_files.

## Additional notes on solenoid configuration in QMK:

For xwhatsit firmware, you need to follow the xwhatsit manual to turn on the solenoid in the xwhatsit capsense usb utility program.  To enable a key to turn it on and off:  as posted by forum members on Deskthority, the solenoid switch is found as “Exp Toggle” in the keymap dropdown in xwhatsit capsense software. I placed it on a layer so I could turn in on and off as I see fit.  For QMK you just need to choose the “Any” key and then type HPT_TOG or any of the other QMK options.  If you do not know how to do this you’ll need to review the QMK manual for more information.

| Name | Description |
|———–|——————————————————-|
|`HPT_ON` | Turn haptic feedback on |
|`HPT_OFF` | Turn haptic feedback on |
|`HPT_TOG` | Toggle haptic feedback on/off |
|`HPT_RST` | Reset haptic feedback config to default |
|`HPT_FBK` | Toggle feedback to occur on keypress, release or both |
|`HPT_BUZ` | Toggle solenoid buzz on/off |
|`HPT_MODI` | Go to next DRV2605L waveform |
|`HPT_MODD` | Go to previous DRV2605L waveform |
|`HPT_CONT` | Toggle continuous haptic mode on/off |
|`HPT_CONI` | Increase DRV2605L continous haptic strength |
|`HPT_COND` | Decrease DRV2605L continous haptic strength |
|`HPT_DWLI` | Increase Solenoid dwell time |
|`HPT_DWLD` | Decrease Solenoid dwell time |

- Buzzer/beeper specifications:  per a Deskthority forum member:  “You want to look for a low powered mini speaker rated around 0.5 W (or lower) and maybe 8Ohm.”  And here’s a post on buzzers/beepers mentioning 50 ohm 0.5w seeming to work for one user:  https://deskthority.net/viewtopic.php?p=463031#p463031 
A video of the beeper functioning on a new Model F:  https://www.youtube.com/watch?v=VIwJYRNhpck

## Additional Resources

Please check out the excellent, comprehensive 300+ page Model F manual project being coordinated by Deskthority forum member darkcruix – as a note this project is comprehensive to all Model F keyboards and is not specific to this project, so do not rely on it for the most up to date firmware and other details:  www.bucklingspring.com

## NKRO note as relayed to me

Why there is not NKRO by default: When the USB Human Interface Device protocol was designed, they had a limit of 6 keys in there. This is the minimum all systems agree on (including BIOS etc.). NKRO is not standardized and there is a good chance, the keyboard doesn’t work in certain circumstances. In fact, on a Mac you can’t get into the recovery mode or NVRAM. On a PC, it depends on the UEFI designer, but I have seen problems with HP laptops.

## Deprecated xwhatsit firmware-specific instructions:

One of the main reasons for the two additional beta firmware options above was that a number of folks had issues with the xwhatsit requirement to manually set the threshold value as the auto setting did not work. If they switched the keyboard around to different computers the threshold value may not always be 100% good with xwhatsit firmware. The other firmwares improved upon xwhatsit by offering a working auto-calibration of the voltage threshold and improving the auto-calibration into many calibration zones on the keyboard as opposed to xwhatsit’s requirement to have one voltage threshold for all keys.

Download the latest xwhatsit firmware

Link to download open source xwhatsit software, firmware, and PCB files

xwhatsit describes the technical underpinnings and background of his project

xwhatsit’s thorough manual and guide to using his controllers

Linux precompiled file (if you have trouble compiling)

The xwhatsit utility works for the latest Mac systems too, as of mid-2020 (you may need to press ctrl+click on the program and click open or do some other security procedures to run on a Mac).  No compiling needed!

Flashing firmware on a Mac:

This guide may offer some help: https://www.instructables.com/DFU-programmer-on-Mac-OS-X/

Flashing firmware on Linux:

A guide from a geekhack forum member:

Follow the directions in the YouTube video for using the beta QMK configurator site to customize the layout and compile new firmware, up to downloading the new firmware.
Install dfu-programmer. For Fedora 34:

dnf install dfu-programmer

Enter keyboard bootloader. On the keyboard, hit: + +

Check if bootloader is detected. Look for a USB device listed as “atmega32u2 DFU bootloader” (using another keyboard):

lsusb

Erase and flash new firmware (the hex file downloaded from configurator site):

sudo dfu-programmer atmega32u2 erase
sudo dfu-programmer atmega32u2 flash /path/to/fw.hex
sudo dfu-programmer atmega32u2 launch

Disconnect, then reconnect keyboard.

- Extract the capsense utility folder linked to above. Open the ibm capsense utility. Click tools – gui keyboard unlock.

- Make sure all your keys are installed correctly (see my YouTube videos for the proper installation procedure, especially the below one). https://www.youtube.com/watch?v=xEm2mewsmrA Set the threshold to 122. Does your grid look like the above grid, with two or three gray dots in the rightmost column? Then you may be good to go.

- Do you see a random mess instead, like what’s pictured here? Your threshold is too low. For most cases it should be between 122 and 130 for Brand New Model F Keyboards. Factory default is 122 but if you’re seeing the above at 122 then you probably need a higher threshold value. Important note – always click store override in EEPROM after you adjust a value; otherwise it’s not saved and you’ll run into the same issue when you disconnect and reconnect your keyboard, or when you restart your computer. Do not touch Auto-calibrate as it does not work for some reason.

- Click Column skips. For F77 select columns 11-15 which are not used. For F62 select columns 9-15. Then click store in EEPROM. Press each physical key on your keyboard a few times and make sure one and only one dot (the same dot for a particular key) turns gray each time you press it. If two or more dots activate when pressing one key, maybe increase the threshold by one value until it does not do that. Close the utility, unplug the keyboard, wait a few minutes, and then plug it back in. Do the same testing again. Also try testing it on another computer and another USB port of your current computer if you’d like to be 100% sure (do both of those options). If the threshold is too high, a key press may not result in any gray squares lighting up – that is the problem if you set the threshold too high – not all your keys would be recognized.

- When 100% sure every key activates, click file, import to import the layout.

- Select the layout based on your configuration.

- You can change specific keys by adjusting them in the drop down menus on the Base Layer, Layer 1, etc. tabs. More details are available in the xwhatsit manual linked to earlier.  A note for those updating their function layers and keyboard layouts: the Fn key should have Fn1 selected in the drop down menu on both the Base Layer and Layer 1 of the xwhatsit GUI. If it is not selected in Layer 1 as well, you may get unwanted key presses.

- Press the left ctrl key on your keyboard and one of the boxes should turn gray, so you know that you are adjusting that key. Then click the box and you can see the drop down menu where you can select an alternative key. Very important to always use your mouse to click inside the xwhatsit window’s big empty space somewhere so you do not inadvertently change the selection after making it. That’s it! Your keyboard should be properly configured with your keyboard now.

- One can load a layout file for keyboards with no keys using a regular voltage threshold (122-130) by keeping the keyboard upside down in the foam.